Documentation for ZPLOT & XZPLOT

--------------------------------

******************************** NOTE ***************************

This documentation is not 100% up-to-date or complete. Also, the

description below of the z-format header word format is incorrect.

Look at the code inside the file readf.f of zplot to see the

correct hearder word format.

*****************************************************************

An interactive plotting and picking program for seismic data

Date:       June 1997

Written by:

----|---/---------------------------------------------------------------

|  /

\ /     Colin A. Zelt                             tel:(713) 527-4757

V      Rice University                           fax:(713) 285-5214

@@ " ^  Dept of Geology and Geophysics, MS-126   home:(713) 799-1228

/    /  6100 Main Street

/__  /   Houston, Texas              email: czelt@geophysics.rice.edu

\- |   USA, 77005-1892             room:  205B

\ |

www: "http://zephyr.rice.edu/department/faculty"

------------------------------------------------------------------------

The main features of this program are:

- interactive plotting, picking, filtering and data editing

- data in z-format format, may be real*4 or integer*2 (obtained

from SEGY format using the program SGY2Z)

- plotting in reduced time versus offset, model position, azimuth

or trace sequential number

- any reducing velocity or unreduced

- trace-normalized (common-maximum-amplitude), true-relative-amplitude

or time-varying-gain amplitude scaling

- bandpass filtering

- power spectra estimation

- trace shading

- overlay traveltime curves

- two simple automatic pickers

- uniras or x-window graphics allowing bw/colour postscript

hard copies

-----------------------------------------------------------------------

Table of Contents:

------------------

1. data format			 8. keyboard input

2. files		 	 9. warning bell

3. graphics menu		10. current array sizes

4. mouse buttons		11. size of graphics window and plot

5. MENUPAR namelist		12. automatic pickers

6. PLTPAR namelist		13. example input files

7. AXEPAR namelist		14. program HEADUP

1. Data format:

The file header words contain the following:

1 - number of traces in file

2 - number of points per trace

3 - sampling interval in microseconds

4 - trace start time in milliseconds

5 - trace end time in milliseconds

6 - number of records in file

7 - number of pick words

The trace header words contain the following:

1 - shot station or number

2 - trace sequential number within record

3 - receiver station or number

4 - data type flag for selection of traces to be plotted

5 - dead trace flag (to omit unwanted traces)

6 - shot-to-receiver offset in metres

7 - receiver-to-source azimuth in minutes of arc

8 - multiplicative gain factor

9 to 9+npick-1 - arrival picks in seconds (npick is determined

from file header word 7)

All header words are integer*4 except trace header words 6 and 7 which are

real*4. The data may be either integer*2 or real*4 (see description of

parameter ifmt in section 6 to choose the data format type).

Records do not have to consist of equal numbers of traces, but the data

must be sorted into gathers, also called records. Records can be

common-shot or common-receiver, but for the purposes of this documentation,

the common parameter will be referred to as "shot". Therefore, for example,

the first trace header word will actually be the receiver station or number

in the case of common-receiver data. Traces will be assigned a negative or

positive offset depending on whether the receiver station number is less

than or greater than the shot station number if ilr=1 or -1 (see

section 6).

Note 1: the actual trace headers are NOT modified, only the picks and

dead trace flag in the active header file (hfile) are changed

using the program HEADUP and the file 'zplot.out' (see

important note in section 14)

Note 2: pick times less than -998 and equal to 0 are flags to indicate

unpicked arrivals; therefore, picks at exactly 0 seconds should

be avoided

2. Files:

executable: 'zplot' (uniras version) or 'xzplot' (x-window version)

input:

dfile           - data file

'zplot.in'      - parameter input file

rfile           - record file containing reducing velocity of

data file and one line of parameters for each

record (optional; described in section 6)

txfile          - traveltimes in "tx.in" format used by the

program RAYINVR (optional)

tpfile          - distance-time pairs (2 columns) for single-    trace picker (optional; see section 12)

hfile           - header file obtained from data file using    program SGY2Z containing header words

(optional; described in section 6)

output:

'zplot.out'     - a log file that contains the changes made to

the pick times and dead trace flags; this file

is used to update the active header file hfile

with the program HEADUP described in section 14.

'z.out'         - data values for selected portions of the traces

in ascii format

Initial input parameters are specified through the file 'zplot.in'.

It is composed of three namelists, menupar, pltpar and axepar. The menupar

parameters may be adjusted in the interactive mode. Other parameters specific

to the data file are contained in the record file.  Examples of these

two files are contained at the bottom of this documentation.

A header file containing certain header words must also be supplied.

Note that only the pick times and dead trace flags (word 5) can be

modified by ZPLOT, all other words remain unchanged.

3. Graphics menu:

The plot parameters contained in the file 'zplot.in' may be

used to obtain an initial plot of the data, after which many of these

parameters can be modified using the menu that appears above

the data in interactive mode (iroute=1). The menu consists of

32 items, 27 of which are plot parameters contained in the file

'zplot.in', the remaining 5 perform special functions. The plot

parameter names appear in the menu along with their current values.

These values may be increased or decreased within allowable limits

by clicking the right or left mouse buttons, respectively, with the

cursor inside the box containing the parameter's name and value.

Note that the updated parameter values will not take effect until the

data are re-plotted using one of several options discussed below and in

sections 4 and 8. The value of most other parameters not included in the

menu can be altered by entering "e" from the keyboard (see section 8)

Special menu functions:

active pick word - the active pick word number is increased or

decreased using the left and right mouse buttons

with the cursor inside the menu box labelled "pick".

The colour of the pick word number corresponds to

the colour of the pick symbols.

The final four menu items will respond immediately to either a left or

right mouse button.

plot - clicking on the menu item "plot" will re-plot the data using

all current parameters in the menu, the left button for a

white-on-black plot, right for black-on-white.

orig/prev plot - clicking on the menu item "orig/prev plot" with the

left mouse button will re-plot the data using the

original values of the minimum and maximum distance

and time (xmin, xmax, tmin and tmax) when the current

record was initially plotted, but all other current

values in the menu. Clicking on the menu item

"orig/prev plot" with the right mouse button will re-plot

the data using the previous values of the minimum and

maximum distance and time (xmin, xmax, tmin and tmax),

but all other current values in the menu. Other options

to re-plot the data are discussed in sections 4 and 8.

auto pick - click the left or right button on the menu item "auto pick"

to perform automatic picking for the active pick word

(the pickers are described in section 12).

quit - click on the menu item "quit" to exit from the program

(a "return" from the keyboard may also be necessary).

4. Mouse buttons:

Within the graphics menu, the function of the left and right mouse buttons

is described in section 3. The function of the middle mouse button and the

left and right mouse buttons inside the limits of the data window (xmin,

xmax, tmin and tmax) are discussed below.

Left: a toggle to remove or re-draw the nearest trace (and associated picks)

to the cursor. When a trace is removed, a small dot appears above its

position on top of the data window as a reminder. These traces will

have the value of header word 5 set equal to minus its current value

whenever the trace is re-moved or re-drawn; thus, the trace will not be

re-plotted when the data window is re-plotted, only when the trace is

re-flagged as "live" by clicking the left mouse button near the offset

indicated by the small dot at the top of the data window. The value of

header word 5 in the file hfile will be updated after running HEADUP

Note: This feature may not work correctly if the values of certain

parameters in the menu have been modified since the last time the data

was re-plotted (i.e., ndecim, clip, amp, sf, rcor, spick, ishade) since

the trace and its associated picks are removed from the screen by

plotting on top of them using the background screen colour (does the term

"bush league" mean anything to you?). Actually, this feature can be

useful for assessing the effect of a bandpass filter or changing other

plot parameters such as ndecim, clip, rcor, etc.; to do this, simply

change the appropriate plot or filter parameters either before or after

removing a single trace and then re-draw the trace using the left button

again.

Right: a toggle to add or remove a pick from the nearest trace to the cursor

associated with the active pick word number (the "nearest trace" must

be plotted, i.e., its dead trace flag is greater than zero).

Note: if the value of tmin does not correspond exactly to a trace

sample point, a small time-shift error (+/- sampling interval/2) will

occur in plotting the trace since the start of each trace is always

the closest sample point to tmin (as a consequence of this, the end

point of each trace may not occur at tmax). However, this error

effects only the plot of the trace; the display and times of picks

are corrected for this time-shift error.

Middle: the cursor positions of two successive clicks of the middle mouse

button define new limit points (xmin, xmax, tmin and tmax) for a

zoom window. This new window will be plotted immediately after the

second click. Note that the cursor positions need not be within the

current data window, so that the "zoom" window may actually represent

an increase in window size. A single click of the middle mouse button

followed by a left or right click outside the data window will

"remove" an unwanted corner for a zoom window.

5. MENUPAR namelist (menu parameters):

iseg - not used in x-window version, xzplot, only used for uniras version,

zplot

irec - record number to plot (first column in record file); if there is no

shot station corresponding to this record number in the record file,

the title of the plot will indicate this error and a new record number

may be selected; each time a new record number is selected, the

menu item "plot" should be used first after selecting the appropriate

xmin and xmax values since "orig/prev plot" will make use of the

xmin and xmax values from the previous record which may not be

appropriate

itype - plot only those traces whose header word 4 is equal to itype; if

itype=0, all traces are plotted  (default: 0)

ixaxis - plot traces versus offset (ixaxis=0 or 1), position (model distance)

(ixaxis=2), azimuth using values between 0 and 360 degrees as

specified in header word 7 (ixaxis=3), azimuth using values

between -180 and 180 degrees by converting those header word 7

values greater than 180 to values between -180 and 0 degrees

(ixaxis=4), or trace sequential number (ixaxis=5); the position

of the shot (model distance) is obtained from the second column

of the record file (rfile) if specified, or 0 otherwise

(default: 0)

Note: if ixaxis is negative, then trace plotting is the same as

when using abs(ixaxis), except the values of xmin and xmax are

automatically set so as to include all traces for the current

record being plotted; the value of ixaxis is reset to its positive

value if the data is re-plotted using the same value of ixaxis for

the same record, i.e., the same value of irec.

iscale - equals 0 for trace-normalized scaling (using parameter amp),

1 for true-relative-amplitude scaling (using parameters

rcor, sf and possibly amp), and 2 for time-varying gain

(using parameters tvg and pvg)  (default: 0)

imute - equals 1 for a back mute (all samples greater than the mute time are

zeroed) and -1 for a front mute (all samples less than the mute

time are zeroed); mutes times are obtained from a single linear

mute function specified by vmute and tmute or input interactively

(see section 8)  (default: 0)

amp - maximum trace amplitude for trace-normalized scaling (iscale=0)

in units of km  (default: 1.0)

Note: amp is also used if iscale=1 and sf=0.0, in which case a value

of sf will be calculated assuming a maximum amplitude of amp for the

first trace plotted; this calculated value of sf will be written to

the window in which ZPLOT is being run and the graphics menu

rcor - if iscale=1, trace amplitudes are scaled by a factor proportional

to their (offset(km)/100)  raised to the rcor power  (default: 0.0)

sf - if iscale=1, sf is a global scale factor for the entire section

(default: 0.01)

clip - the amplitude (km) at which clipping is applied; if clip=0, there

is no clipping  (default: 0.)

vred - reducing velocity (km/s) to apply to the data before plotting; this

value is NOT in addition to the amount the data are reduced in the

data file, i.e., if vred equals the reducing velocity already applied

to the data file, no further reduction is applied  (default: reducing

velocity applied to data file)

itx - equals 1 or -1 to overlay theoretical traveltime curves in file txfile;

if itx=-1, the sign of the offsets will be changed from positive to

negative and vice versa; only those traveltime curves corresponding to

the shot with the same model position as that specified in the record file

(rfile) for the current record will be plotted; itx=2 will overlay the

pick template when itemp=1 is used  (default: 0)

nskip - plot every nskip'th trace  (default: 1)

ndecim - plot every ndecim'th data point  (default: 1)

ishade - equals 1 (-1) for shading plus trace or 2 (-2) for shading

only with the positive (negative) peaks shaded  (default: 0)

Note: the shading is achieved by plotting horizontal lines

within the appropriate peaks

spick - length in mm of pick symbols (arrows); if spick=0, no picks

are plotted and no picking is allowed until the data are re-plotted

with spick>0; the menu items "auto pick" and the active pick header

word number will not appear if spick=0 the last time the data was

re-plotted; these items will also be disabled whenever spick is set

to zero after the data has been plotted  (default: 0.)

ibndps - apply a Butterworth frequency bandpass filter to the data

(ibndps=1) or calculate the power spectra of each trace and

apply the bandpass filter (ibndps=-2) or calculate only the

power spectra (ibndps=-1)  (default: 0)

Note: see the book "Time sequence analysis in geophysics", 3rd

edition (pp. 237-280) by Kanasewich for details of the bandpass

routine, or D. White, the man who coded this amazing subroutine;

see the book "Numerical Recipies" by Press et al. for details of

the power spectra routine.

izerop - apply the Butterworth bandpass filter as a zero-phase filter

(default: 1)

freqlo, freqhi - low and high cut frequencies of the Butterworth filter;

(defaults: 1., 20.)

tlinc - time increment (s) to plot horizontal time lines; if tlinc

equals 0.0, no time lines are plotted  (default: 0.)

xmin, xmax - minimum and maximum distance (km) of plot; only traces with

offsets in this range are plotted; negative values must be

used to plot traces with a station number less than the shot

station number; xmin must be less than xmax always

xmm - length of distance axis in mm  (default: 230)

tmin, tmax - minimum and maximum time (s) of plot; tmin must be less than

tmax always, even if itrev=1

tmm - length of time axis in mm  (default: 130)

6. PLTPAR namelist (plotting and picking parameters):

dfile - data file; must be sorted into gathers (also called records)

rfile - record file corresponding to data file; contains the reducing

velocity (km/s) of the data file on the first line followed

by a line consisting of 4 columns for each record, each

line corresponding in order to the records in the data file (free

format is used). The 4 columns contain the record number (can be

different from shot station number in active header file), shot

position in model (km), azimuth of line (degrees) and the title of

the plot.

Note: the first column (record number) must correspond to the value

of irec in the menupar namelist and must be a positive integer

(default:  if rfile not specified or does not exit, the reducing

velocity is assumed to be 0 km/s, shot numbers are assigned

sequentially from 1, shot position and azimuth are assumed to be 0,

and the title is blank)

hfile - header file obtained from data file using the program SGY2Z containing

header words and the pick words; this file may be over-written

with the current picks and dead trace flags at the end of each run

using the program HEADUP; see important note in section 14

(default: if hfile is not specified or does not exist, the shot

station number is 1 for all traces, the receiver station number

equals the trace sequential number, the data type flag (header

word 4) is 1 for all traces, offset and azimuth are 1 metre and 0

degrees for all traces, the dead trace flag is 1 for all traces,

and all pick times are undefined)

txfile - file containing theoretical traveltime curves output by RAYINVR

to be overlain if itx=1 or -1; traveltimes must be unreduced

tpfile - file containing the pick template used by the single-trace picker

if itemp=1. The file contains two columns of numbers (free format)

where the first column is source-receiver offset (km) and the

second is unreduced time (s); the first offset should be zero

and the remaining offsets should monotonically increase. The pick

template will be plotted over the data when itx=2 (see section 12).

iroute - not used in x-window version, xzplot, only used for uniras version,

zplot

xwndow, ywndow - the horizontal and vertical size of the graphics window

in mm that will over-ride a window size specified in

an "mx11.opt" or ".Xdefaults" file; the maximum allowable

values (to use the entire screen) may be about 290 and 220 mm,

respectively  (defaults: if xwndow or ywndow is

less than or equal to zero, the size of the window specified

in the "mx11.opt" or ".Xdefaults" file is used)

ibw - equals 1 to plot black traces on white background, otherwise, plot is

white on black; this parameter is adjusted by clicking on the "plot"

menu item using the left (ibw=1) and right (ibw=0) buttons as described

in section 4.  (default: 1)

Note: if iseg=1, ibw=0 is automatically used

ifmt - data trace values in the file hfile are integer*2 (ifmt=0) or

real*4 (ifmt=1)  (default: 0)

icomp - plot only those traces whose header word 5 is equal to icomp; if

icomp=999, all traces are plotted  (default: 999)

ilr - if ixaxis=0, 1 or 2, plot the traces with a corresponding receiver

station number less than the shot station number with a negative offset

if ilr=1, or vice versa if ilr=-1; use the azimuth of the line specified

in the 4th column of the record file (rfile) and the trace azimuth in the

active header file (hfile) to determine positive or negative offset if

ilr=0; if ixaxis=5, then plot trace sequentially from right to left if

ilr=-1, otherwise plot from left to right  (default: 1)

iscreen - plot some trace header values to the window in which ZPLOT is being

run for each trace plotted  (default: 0)

iout - output portions of the traces centred about the pick locations to the

file 'z.out' in ascii format; the DC value of the trace (within the

plotted time window) will not be removed if iout=2; the parameters

nout and nbout determine the start time and window length to be

output  (default: 0)

Note: only traces which have a pick within the plotted time window

for the active pick word will be output

overlap - the overlap (percentage) of the current x-axis range (defined by

xmin and xmax) with the new x-axis limits when entering "l" or "r"

from the keyboard (see section 8); overlap must be between 0 and 90

(default: 10)

dens - number of lines per mm for trace shading  (default: the pixel

resolution of the device is calculated to determine an appropriate

value, about 4 for some screens)

colour - an array containing the colours

of the (1) menu boundaries, (2) special function menu items,

(3) traveltime curves contained in the file txfile, (4) pick

template contained in the filie tpfile, and (5) mute function

obtained by using imute=1 or -1 (see sections 6 and 8)

(default: 4, 3, 6, 4, 5; blue, green, pink, blue, light blue)

Note: if colour(1)=-1, then all plotting will be black and white,

either white on black (ibw=0) or black on white (ibw=1).

pickc - an array containing the colours

of the pick symbols corresponding to the pick words

(default: 2, 3, 4, 5, 6; red, green, blue, light blue, pink)

xpick - shift in mm (positive right, negative left) to plot pick

symbols beside trace if spick>0  (default: 0.)

tcrcor - length (s) of the cross-correlation window for the lateral trace

correlation automatic picker (see section 12)

(default: 0.3)

tlag - cross-correlations are calculated using the lateral trace correlation

automatic picker by shifting the trace to be picked between plus and

minus tlag seconds on either side of an arrival time obtained by linear

interpolation between the two existing picks on either side of the

trace being picked (see section 12)  (default: 0.1)

twinener - length of time window (s) for single-trace picker (see section 12)

(default: 0.1)

minenratio - minimum allowable energy ratio to make a pick using the

singe-trace picker (see section 12)  (default: 10)

itemp - use a pick template file (tpfile) as a guide for the single-trace

picker (see section 12; the pick template may be plotted using itx=2

(default: 0)

tempwin - when using the single-trace picker (see section 12) and itemp=1,

restrict all picks to fall within the time defined by the pick

template file plus or minus tempwin (s)  (default: 1)

npoles - number of poles of Butterworth filter; must be no less than 4

and a multiple of 2  (default: 8)

idot - plot a small dot at the top of the data window to indicate the position

of traces with a negative dead trace flag  (default: 1)

ipicko - produce a pick overlay; only the pick symbols and a box representing

the data window are plotted  (default: 0)

tvg - operator length (s) for time-varying gain  (default: 1.0)

pvg - power of time-varying gain operator; divide each point of the trace by

the sum of the absolute value of the points in a window of length tvg

centred about that point raised to the pvg power  (default: 1.0)

Note: pvg<0 accentuates "highs", pvg>0 brings up "lows" (the usual

purpose of the filter), and pvg=0 has no effect

vmute, tmute - muting velocity (km/s) and intercept time (s) (unreduced) for

a front or back mute depending on the value of imute

(defaults: 0., 0.)

recht - height of receiver station numbers (header word 3) in mm annotated

at the top of the plot above each trace if recht>0  (default: 0)

ndrec - plot every ndrec'th receiver station if recht>0 (default: 1)

title - title of plot (up to 80 characters); if title='' then title

from record file is used  (default: '')

space - space in mm between top of plot and title to allow room for receiver

station numbers if recht>0  (default: 0)

nout - number of trace samples to output if iout>0; if nout=0, the entire

plotted seismogram is output  (default: 0)

nbout - number of samples before the time corresponding to the active pick

word to output if iout>0  (default: 0)

7. AXEPAR namelist (axes parameters):

itrev - equals 1 to plot time axis increasing down (default: 0)

Note: always use tmin < tmax if itrev=0 or 1

albht - height of axis labels in mm  (default: 2.5)

amenht - height of graphics menu items in mm; also controls the

overall size of the menu; if amenht=0, then no menu will be

plotted when iroute does not equal 1  (default: albht)

orig - lower left origin of plot is (orig,orig) mm  (default: 12.5)

ntickx - number of intervals separated by tick marks along distance axis,

i.e., number of tick marks equals ntickx+1  (default: determined

by the program based on the values of xmin and xmax)

xmint, xmaxt - minimum and maximum distance (km) of tick marks

(default: determined by the program based on the values of

xmin and xmax)

ndecix - number of digits after decimal in distance axis annotation;

-1 for integer annotation  (default: determined by the program

based on the values of xmin and xmax)

ntickt - number of intervals separated by tick marks along time axis,

i.e., number of tick marks equals ntickt+1  (default: determined

by the program based on the values of tmin and tmax)

tmint, tmaxt - minimum and maximum time (s) of tick marks

(default: determined by the program based on the values

of tmin and tmax)

ndecit - number of digits after decimal in time axis annotation;

-1 for integer annotation  (default: determined by the program

based on the values of tmin and tmax)

fmax - maximum frequency (Hz) of power spectra plot  (default: the Nyquist

frequency = 1/(2dt) where dt is the sampling interval)

8. Keyboard input:

There are several lower-case letters and numbers that can be input from the

keyboard to provide additional interactive functions either not available

in the menu or as an alternative means of activating them in the case of

certain menu functions.

a - replot the data by aligning the traces that have been picked for

the active header word; the picks are aligned at the time position

where the cursor is when "a" is entered. This feature is useful

for checking the lateral correlation of the waveforms of the picked

arrivals.

d-d - two successive entries of "d" will cause all picks for the active

pick word to be deleted between the cursor positions when the two

entries were made. This provides a quick means of removing many

unwanted picks if, for example, the automatic picker fails.

e - edit the value of input parameters that are not included in the

menu; this allows virtually all program parameters to be changed

interactively without having to exit the program; the user is

prompted for the parameter and its new value from the window in

which ZPLOT is being run.

h - write to the window in which ZPLOT is being run the trace header

values of the trace closest to the cursor position

l,r - pan left (l) or right (r) by re-plotting with the values of

xmin and xmax adjusted so that their difference remains the

same and the overlap of the x-axis range with the current

screen is determined by the parameter overlap (see section 6).

m-m - the two end points defining the mute function may be input

interactively by entering "m" for the two successive cursor

positions; the new mute function will be indicated by a

line and the new values of vmute and tmute will be written to

the window in which ZPLOT is being run

Note: this feature may also be used to determine the apparent

velocity and intercept time of observed phases using the

values of vmute and tmute that are written to the window

o - re-plot the data as you would be clicking the menu item "orig"

(i.e. re-plot the data using the original values of the minimum

and maximum distance and time (xmin, xmax, tmin and tmax) when

the current record was initially plotted, but all other current

values in the menu)

p - re-plot the data as you would by clicking on the menu item "plot"

(the plot colour format, i.e., black-on-white or vice versa, remains

the same)

q -  quit from the program as you would by clicking on the menu item

"quit"; this option for quitting may be useful if, for example, the

window size determined by xwndow and ywndow (see section 6) is so

large that the menu item "quit" cannot be seen (a "return" from

the keyboard may also be necessary).

s - save the current values of xmin, xmax, tmin and tmax (for use later

when using the letter "t" from the keyboard, see below)

t - replot the data using the values of xmin, xmax, tmin and tmax that

were saved the last time "s" was entered (see above). If "t" is

entered again, the data will be reploted using the values of

xmin, xmax, tmin and tmax when "t" was entered; therefore, by

successively entering "t" from the keyboard, you can toggle between

two different data windows.

z - zoom in by a factor of 2 (the x-axis and time-axis ranges are reduced

by 50%) about a point centred at the cursor position; the method by

which to obtain a zoom window of any size is described in section 4

9. Warning bell:

A bell will sound in seven situations.

1. if the values of tmin, tmax, xmin or xmax are inappropriate

(i.e. xmin>xmax or tmax-tmin is so small that less than 2 points

per trace will be plotted or so large that array dimensions will

be exceeded for plotting or the fourier transform when power

spectra are to be determined).

2. if any of the parameters xmm, tmm, albht, amenht or orig were

adjusted to fit the plot within the graphics window.

3. if the value of sf equals 0 and iscale=1.

4. if no record file (rfile) or header file (hfile) has been specified

or those specified do not exist.

5. if the files specified in txfile of tpfile (see section 6) do not

exist; the user will be prompted for new file names from the window

in which ZPLOT is being run; a "return" from the keyboard will stop

ZPLOT from asking for a valid file name until an attempt is to made

to open either file again.

6. if the pick template file consists of less than 2 points

7. after the menu item "quit" is clicked on (a "return" from the keyboard

may be required to exit the program).

10. Current array sizes:

number of points per trace - 10000

number of traces - 20000

number of records - 1000

number of header picks words - 10

number of points defining pick template - 100

These limits may be adjusted in the file 'zplot.par':

11. Size of graphics window and plot:

The size of the graphics window can be controlled by the input

parameters xwndow and ywndow as described in section 6.

The values of albht, amenht, orig, xmin, xmax,

xmm, tmin, tmax and tmm will automatically be adjusted as necessary in

order to fit the data and menu within the graphics window.

12. Automatic pickers:

Lateral Trace Correlation Picker:

Click on the menu item "auto pick" with the left mouse button

and the automatic picker will attempt to make picks for the active pick

word for those traces currently displayed and bracketed by traces that

have existing picks. The picks are made by performing cross-correlations

at various lags with a distance-weighted average of the two bracketing

traces. The size of the cross-correlation window and the maximum allowable

lag time with respect to a linearly interpolated arrival time are controlled

by the parameters tcrcor and tlag as discussed in section 6. The picker may

fail to make a pick for one of three reasons: (1) the "auto pick" feature is

disabled since spick=0 (see section 6), (2) there is less than tcrcor seconds

displayed after one or both of the bracketing picks in the data window

(increase tmax and re-plot the data), and (3) failure of a reliability test

in which an optimal lag time is also calculated with respect to each of the

bracketing traces and these lags compared to the lag determined from the

distance- weighted average trace. Basically, the bracketing picks should

represent the most reliable or obvious ones and have similar waveforms, and

the arrivals in between should have a fairly linear trajectory between the

bracketing picks.

Single-Trace Picker:

Click on the menu item "auto pick" with the right mouse button

and the automatic picker will attempt to make picks for the active pick

word for those traces currently displayed but not yet picked. The picks

are made by choosing the point along the displayed portion of the trace

corresponding to the maximum value of the ratio of trace energy in a window

of length twinener seconds on either side of the point. Traces for which

this ratio does not reach the value of minenratio will not be picked. If

itemp=1 (see section 6), picks will be restricted to fall within plus or

minus tempwin (s) of the time specified in the pick template file (tpfile)

for a trace at that particular source-receiver offset. The pick template

file may be plotted over the data with itx=2.

The lateral trace correlation picker is generally more robust since the user

must have supplied bracketing picks. The single-trace picker is generally only

good for first arrivals with good signal-to-noise ratio or if a small value of

tempwin is used with itemp=1. Bandpass filtering (ibndps=1) may improve the

performance of the pickers if the data are noisy.

********************************************************************************

*------------------------------------------------------------------------------*

*| Note: ALWAYS check the picks afterwards, the pickers don't work very well! |*

*------------------------------------------------------------------------------*

********************************************************************************

13. Example input files:

zplot.in:

&menupar

iseg=0, ixaxis=0, itype=0, itx=0, ishade=1,

irec=4, vred=4.5, tlinc=1.0,

iscale=0, amp=0.2, sf=0.00512, rcor=1., clip=2.5,

nskip=2, ndecim=2,

ibndps=0, izerop=0, freqlo=1., freqhi=12.,

spick=0.10,

xmin=-20., xmax= 20., tmin=0.0, tmax=5.0,

xmm=335., tmm=180.0,

&end

&pltpar

iroute=1, iscreen=0, idot=1, ilr=0,

overlap=10., xwndow=350., ywndow=230., dens=5.,

dfile='test.dat', rfile='test.rec', hfile='test.hdr',

recht=0., ndrec=1, space=8.,

itemp=1, tempwin=.5, twinener=.2, minenratio=10.,

colour=4,3,6,4,5,

iout=0, nout=100, nbout=10,

txfile='tx.in', tpfile='temp.in'

&end

&axepar

itrev=1,

albht=2.5, amenht=4.0, orig=12.,

fmax=20.,

&end

record file (rfile):

7.0

20   0.00  0.0 'GRAP 88 - SP20'

19  40.27  0.0 'GRAP 88 - SP19'

18  78.76  0.0 'GRAP 88 - SP18'

17 139.38  0.0 'GRAP 88 - SP17'

16 171.66  0.0 'GRAP 88 - SP16'

15 213.73  0.0 'GRAP 88 - SP15'

14 258.59  0.0 'GRAP 88 - SP14'

pick template file (tpfile):

0.000     0.800

1.600     1.500

5.500     3.675

8.500     4.725

12.000     5.545

15.000     6.341

20.000     7.652

26.000     8.900

30.000     9.614

35.000    10.446

40.000    11.269

45.000    12.080

50.000    12.878

55.000    13.590

14. Program HEADUP:

The executable HEADUP is located in /home/scratch/scorpio/zelt/sunbin

and is used to update the active header file hfile after a picking

session. Input is through the file 'zplot.out' and the file hfile. In

addition, the name of the active header file and number of pick words

are input interactively. The active header file is then updated (i.e.

over- written) with the new pick times and dead trace flags.

*********************************************************************

* IMPORTANT NOTE: you must run the program HEADUP after a picking   *

* --------------  session and before beginning another picking      *

*                 session or else the pick changes from the first   *

*                 session (in the file 'zplot.out') will be lost    *

*                 since the file 'zplot.out' is over-written each   *

*                 time ZPLOT is run                                 *

*********************************************************************